# MIEN2208E

8-port 10/100M Layer-2 Din-Rail Unmanaged Industrial Ethernet Switch

Support 8x 100M copper ports for flexible

network

- 10Base-T/100Base-TX and auto MDI/MDI-X

- Plug and play, supports power status relay alarm output

- High strength enclosed aluminum shell with IP40 protection level, fanless shell for heat dissipation, durable, stable and reliable

- Supports DC 12-48V DC power input, dual power redundancy, and reverse protection

- Working temperature -40 ℃~+85 ℃, meeting the requirements of harsh industrial environment applications

# Product Introduction

MIEN2208E is a layer-2 unmanaged industrial Ethernet switch that designed and developed by Maiwe Communication for industrial communication network applications. This switch is mainly used for simple plug and play applications. This series of products provides 8x 10Base-T/100Base-TX Ethernet RJ45 ports. Each RJ45 port has adaptive function, which can automatically configure to 10Base-T or 100Base-TX status and full duplex or half duplex operation mode, and can automatically connect to MDI/MDI-X. To make industrial communication smoother, more reliable, and faster, and meet the continuous innovation needs of customers to improve value-added applications.

The MIEN2208E switch provides a wide range of DC power input, uses industrial din- rail installation. This product has the characteristics of high-speed line speed forwarding and plug and play. It adopts industrial grade quality design solutions on its core devices, ensuring stable operation of equipment in harsh industrial environments and high electromagnetic radiation scenarios. The product is widely used in industrial fields such as comprehensive energy sources, smart cities, rail transit, intelligent transportation, and industrial automation.

| Technical specifications |  |
| --- | --- |
| Switch capability |  |
| Backboard bandwidth | 1.6Gbps |
| Cache size | 768Kbit |
| MAC Table Size | 2K |
| Interface |  |
| 100M port | 8x 10Base-T/100Base-TX Ethernet RJ45 ports |
| relay alarm | 1 relay alarm output, using 5.08mm 6PIN wiring terminals,occupying 2 Pins |
| Indicator lights | Power light, operation light, port rate and connection/activity light |
| Power |  |
| Power Input | DC 12-48V dual power input |
| Full load power | ＜1.8W@DC24V |
| Connection | 5.08mm 6PIN wiring terminals,occupying 4 Pins |
| Power protection | Reverse polarity protection |
| Physical Characteristics |  |
| Dimension | 95×43×90.5(mm) (not including the din-rail) |
| Installation | Standard 35mm DIN-rail Mounting |
| Shell protection | IP40 |
| Weight | About 0.34kg |
| Working Environment |  |
| Operating temperature | -40℃~ +85℃ |
| Storage temperature | -40℃~ +85℃ |
| Ambient humidity | 5%~95%（No condensation） |
| Industry Standard |  |
| EMC protection | IEC 61000-4-2(ESD):Level 4(Contact discharge±8kV,air discharge±15kV) IEC 61000-4-5(Surge):Level 3 (Power:common mode±2kV，differential mode±1kV network port:common mode±6kV,differential mode±2kV） IEC 61000-4-4(EFT):Level 4(Power:±4kV,Copper port:±2kV) |
| Certification | CE,FCC,RoHS |

# Dimension drawing

Unit:mm

# Ordering Information

| Model | 100M copper port | Power range |
| --- | --- | --- |
| MIEN2208E | 8 | Dual DC 12~48V |

|  | # Contact Us |  |  |  |
| --- | --- | --- | --- | --- |
| Wuhan Maiwe Communication Co., Ltd |  |  |  |  |
| Address: No.52 Liufang Avenue, East lake High-tech Development Zone, Wuhan, China. Tel: 027-87170215/16 Fax: 027-87170217 Mail: enquiry@maiwe.com Official site: www.maiwe.com |  |  |  |  |
| Copyright © Maiwe Communication All rights reserved. |  |  |  |  |